Uconfig let one workstation with all kinds of IP to browse the compex device.

In the compex device: I use Mac Hook to receive/send uConfig packets.

In the workstation(windows): I use socket to receive/send uConfig packets.

| Workstation (for example IP: 192.1.1.1 Mac: xx-xx-xx-xx-xx ) Send NPFind request packet (192.1.1.1.255) ---------------------------------------- Then Workstation Enter uConfig mode( (a) add one host route item in route table( use deceiving host route 192.1.1.2 for 66.2.2.2, (b) add one deceiving arp entry yy-yy-yy-yy-yy for the deceiving gateway 192.1.1.2 in the arp table) . Send Begin_uConfig command to compex device with unicast packet Now workstation can browse the compex device’s web server |     | Compex device (for example 66.2.2.2 Mac: yy-yy-yy-yy-yy ) --------------------------------- Respond the NPFind packet with unicast packet( For device works in mac level, so it can send the unicast packet to workstation even not same subnet with workstation) ---------------------------------------- If compex device allow to enter uConfig Then Enter uConfig mode ( (a) add one host route item in route table( use deceiving host route 66.2.2.1 for 192.1.1.1, (b) add one deceiving arp entry xx-xx-xx-xx-xx for the deceiving gateway 66.2.2.1 in the arp table) . Then send Begin_uConfig_ACK to workstation with unicast packet. If not allowed to enter uConfig mode, then send Deny packet to it with unicast packet |
| --- | --- | --- |