The method to add uconfig for other device (mainly according to NP15B):

There are only two file uconfiglib.h and uconfiglib.c for the uconfig function.

1) add uconifg.c to your  project or link LibUconfig.a; copy uconfiglib.h into your project directory.

2) modificaiton in  cfg.h

a) add #include "uconfiglib.h"

b) add struct UconfigData uCfgData;  at the last of the struct of CONFIG_FILE

3) modificaiton in  start.c

a)  when Lan ip stack is setup and configfile is read, use the fucntion:

void ReadUconfigData( struct UconfigData *puCfgData, int bfColdReBoot);

BOOL SetUconfigWithLanPort(char *DeviceName, int unit,

char *LanIP, char *Mask,

ULONG product_ID,

char *ProductNameWeb,

char *sysName);

for example in NP15B, after below program :

if (ipAttach(LAN_DEVICE_UNIT, LAN_DEVICE_NAME) != OK)

{
             printf ("Fatal Error: Failed to attach to lan interface\n");
             return ERROR;
         }

add the programs:

/*sgh add BEGIN: to support uconfig/one-to-one mode */

taskLock();

SetUconfigWithLanPort("ksm", 0, configFile->intip, configFile->intmask,

PRODUCT_ID,

SYSMODELWebName(),

configFile->sysname);

if( getRebootTime() == 0) /*cold reboot */

SetUconfigDataMemAddr_Read( &configFile[0].uCfgData, 1);

else /*warm reboot */

SetUconfigDataMemAddr_Read( &configFile[0].uCfgData, 0);

taskUnlock();

4) httpposts.c ( for the chaning of lan port's IP and MASK)

a) add #include "uconfiglib.h"

b) when lan port mask is changed, must use function  ReNewLanPortattr(..)

for example in NP15B, modify the fuction httpPostIntip(...) as below when lan port mask is changed:        ....

if( strcmp (configFile->intmask, mask) != 0 )  {
              strcpy (configFile->intmask, mask);
              ifMaskSet(LAN_DEVICE, ntohl(inet_addr(mask)));

ReNewLanPortattr(configFile->intip, mask);/*sgh add to support one-to-one mode*/

dhcpd_update_needed = 1;
    }

c) when lan port ip is changed, must also use function  ReNewLanPortattr(..) at last. But in

NP15B, when ip is changed, a new task in function httpPostIntip(...) is created  as below:

taskSpawn ("setip", 30, 0, 1024*12, (FUNCPTR)CMDSetIntIpTask, (int)ip, (int)mask, 0, 0,

0, 0, 0, 0, 0, 0);

so the real place to add ReNewLanPortattr(...) is in the function CMDSetIntIpTask(...) which is

defined in cmd.c. The new CMDSetIntIpTask(...) should be as below:

void CMDSetIntIpTask(pCHAR ip ,pCHAR mip)
        {
                taskDelay(60);
                HTTPDShutDown();
                taskDelay(10);
                CMDSetIntIP(ip, mip);
                CFGWriteConfig();

/*sgh add Begin: to support uconfig/one-to-one mode*/

taskDelay(10);

ReNewLanPortattr(ip, configFile->intmask);

/*sgh add End: to support uconfig/one-to-one mode*/

5) http.c  ( to support not different subnet ip )

delete bleow program in the function httpDaemon(...)

if ( !inet_same_subnet (LAN_DEVICE, clientAddr.sin_addr) )  {
            close (newSock);
            continue;
    }

6) msglog.c      (If someone have made some modification in this file from NP15B V2.74, maybe below program is not correct )

Add:

#define MSGLOG_MSG_UCONFIG  1 /*Sgh add*/

- Modify:

#define MSGLOG_MSG_BLOCK     1

to

#define MSGLOG_MSG_BLOCK     3 /*sgh modify it to 3 from 1*/

- Add:

static char *NormalMsg = "Message Window"; /*sgh add*/

- Modify below program in char *msglog_htm_dynamic[]

From:

Message Window

To

“, NULL, “

- Add before msglog_htm_dynamic[MSGLOG_MSG_BLOCK] = msglogBuffer in function  update_msglog_htm (…):

/*sgh add Begin:   for uconfig */

if( IsUconfigMode() )

{

msglog_htm_dynamic[MSGLOG_MSG_UCONFIG] = Uconfig_WARN_STR; /*sgh */

}

else

{

msglog_htm_dynamic[MSGLOG_MSG_UCONFIG]= NormalMsg;

}

/*sgh add End:   for uconfig */

7) httpd.c: for wan port’s dynamic ip when clone mac.

a) add #include "uconfiglib.h"

b) before  “return etherAddrResolve(pIf, targetAddr, enetAddr, 1, 1)” add below program:

/* Sgh add */

if( IsUconfigMode() )

{

Get_WS_MAC( enetAddr);

return OK;

}

Modification reason: it seems that etherAddrResolve(…) not support different subnet.

8) httpfiles.c

a) #include "uconfiglib.h"    /*sgh*/

b) add :

{"/ExituConfig.htm", ExitUconfig_htm, sizeof(ExitUconfig_htm),  (FUNCPTR)ExitUconfig},  /*sgh for ExitUconfig*/

9) menu.c( for add ExitUconfig button)

(If someone have made some modification in this file from NP15B V2.74, maybe below program is not correct )

a) #define MENU_EXIT_UCONFIG    5  /*Sgh add*/

b) in menu_htm_dynamic[]

modify

<table border=\"0\" width=\"100%\" cellspacing=\"0\" cellpadding=\"0\" bgcolor=\"#ffffff\">\n\

to

<table border=\"0\" width=\"100%\" cellspacing=\"0\" cellpadding=\"0\" bgcolor=\"#ffffff\">\n",

NULL,

"<TD width=30% align=left><font face=Arial>,\n

c) in function int update_menu_htm (void)  add:

/*sgh add for uconfig*/

if( IsUconfigMode() )

menu_htm_dynamic[MENU_EXIT_UCONFIG] = UconfigUrl;

else

menu_htm_dynamic[MENU_EXIT_UCONFIG] = "";

/*sgh add end*/

before original program

/* Update the file */

if ( httpFilesForm ( &menu_htm, &file_size, menu_htm_dynamic) != OK )  {…

That is all for the modification.